# Mastering Embedded System

# Online Diploma

www.learn-in-depth.com

# First Term (Final Project 1)

# Eng. Omar Khaled M. Anwer

# My Profile:

# Case Study

Specifications:

- A pressure controller should inform the crew of a cabin
with an alarm when the pressure exceeds 20 bars in the cabin.

- The alarm duration equals 60 seconds.

- The system keeps tracking the measured values.

Pressure Controller Assumptions

- The controller set up and shutdown procedures are not modeled.

- The controller maintenance is not modeled.

- The pressure sensor never fails.

- The alarm never fails.

- The controller never faces power cut.

Versioning:

The “keeps tracking the measured values” option is not modeled in the first version of the design.

# Requirement Diagram

# System Analysis

# Use Case Diagram

# Activity Diagram

# Sequence Diagram

This sequence diagram describes a scenario of the system.

# System Design

# Application

# Pressure Sensor

# Alarm

# Compilation

# Makefile

#@copyright : Omar Anwer

#Project name

PROJECT_NAME	= Pressure_Controller

#Architectures Specific Flags

CPU 			= cortex-m3

CFLAGS_ARCH 	= -mcpu=$(CPU)

#CFLAGS_ARCH    = -mcpu=$(CPU) -m$(ARCH) --specs=$(SPECS)

#Compiler Flags and Defines

CC				= arm-none-eabi-

DBGCFLAGS		= -g -gdwarf-2

CFLAGS			= -ansi -std=c89 -O0 -Wall $(CFLAGS_ARCH) $(DBGCFLAGS)

#Linker Flags

LINKER_FILE		= linker_script.ld

LDFLAGS_ARCH 	= -T $(LINKER_FILE)

STARTUP_FILE	= startup.s

#includes

INCS			= -I .

LIBS			=

#.c and .s files

SRC				= $(wildcard *.c)

AS				= $(wildcard *.s)

PRE				= $(SRC:.c=.i)

SRCOBJ			= $(SRC:.c=.o)

ASOBJ			= $(AS:.s=.o)

all: $(PROJECT_NAME).bin

@echo ""

@$(CC)size.exe $(PROJECT_NAME).elf

@echo ""

@echo "Building done..."

%.o: %.s

$(CC)as.exe $(INCS) $< -o $@

@echo ""

%.o: %.c

$(CC)gcc.exe -S $(INCS) $<

@echo ""

$(CC)gcc.exe -c $(CFLAGS) $(INCS) $< -o $@

@echo ""

%.i: %.c

$(CC)gcc.exe -E $(INCS) $< -o $@

@echo ""

$(PROJECT_NAME).elf: $(ASOBJ) $(SRCOBJ) $(PRE)

$(CC)ld.exe $(LDFLAGS_ARCH) $(LIBS) $(ASOBJ) $(SRCOBJ) -o $@ -Map=$(PROJECT_NAME).map

@echo ""

$(CC)objdump.exe -h $@

@echo ""

$(CC)readelf.exe -S $@

@echo ""

@cp $(PROJECT_NAME).elf $(PROJECT_NAME).axf

$(PROJECT_NAME).bin: $(PROJECT_NAME).elf

$(CC)objcopy.exe -O binary $< $@

@echo ""

$(CC)objcopy.exe -O ihex $< $(PROJECT_NAME).hex

@echo ""

clean:

@rm *.bin *.hex *.elf *.axf *.map

clean-all:

#@rm  -rf $(filter-out $(STARTUP_FILE), $(AS))

@rm *.s

@rm  *.i *.o *.bin *.hex *.elf *.axf *.map

@echo "All cleaned..."

# Startup code

#include <stdint.h>

/*#define stack_top	0x20001000*/

extern int main(void);

extern uint32_t _stack_top;

extern uint32_t _E_text;

extern uint32_t _S_data;

extern uint32_t _E_data;

extern uint32_t _S_bss;

extern uint32_t _E_bss;

void Reset_Handler(void)

{

/* copy .data section byte by byte from FLASH to SRAM */

uint8_t* pSrc = (uint8_t*)(&_E_text);

uint8_t* pDst = (uint8_t*)(&_S_data);

uint32_t DATA_SIZE = (uint8_t*)(&_E_data) - (uint8_t*)(&_S_data);

uint32_t i;

for(i = 0; i < DATA_SIZE; ++i)

{

*pDst = *pSrc;

pSrc++;

pDst++;

}

/* initialize .data section in SRAM */

pDst = (uint8_t*)(&_S_bss);

uint32_t BSS_SIZE = (uint8_t*)(&_E_bss) - (uint8_t*)(&_S_bss);

for(i = 0; i < BSS_SIZE; ++i)

{

*pDst = 0;

pDst++;

}

/* jump to main() */

main();

}

/* initialize vectors */

void Default_Handler(void)

{

Reset_Handler();

}

void NMI_Handler(void)			__attribute__(( weak, alias("Default_Handler") ));

void H_fault_Handler(void)		__attribute__(( weak, alias("Default_Handler") ));

void MM_fault_Handler(void)		__attribute__(( weak, alias("Default_Handler") ));

void Bus_fault_Handler(void)	__attribute__(( weak, alias("Default_Handler") ));

void Usage_fault_Handler(void)	__attribute__(( weak, alias("Default_Handler") ));

uint32_t vectors[] __attribute__((section(".vectors"))) = {

(uint32_t) &_stack_top,

(uint32_t) &Reset_Handler,

(uint32_t) &NMI_Handler,

(uint32_t) &H_fault_Handler,

(uint32_t) &MM_fault_Handler,

(uint32_t) &Bus_fault_Handler,

(uint32_t) &Usage_fault_Handler,

};

# Linker Script

/*

Linker script Cortex-M3

By Eng.Omar

*/

ENTRY(Reset_Handler)

MEMORY

{

FLASH (rx) : ORIGIN = 0x08000000, LENGTH = 64K

SRAM (rwx) : ORIGIN = 0x20000000, LENGTH = 20K

}

SECTIONS

{

.text :

{

*(.vectors*)

*(.text*)

*(.rodata)

. = ALIGN(4);

_E_text = .;

}>FLASH

.data :

{

_S_data = .;

*(.data)

. = ALIGN(4);

_E_data = .;

}>SRAM AT>FLASH

.bss :

{

_S_bss = .;

*(.bss*)

_E_bss = .;

. = ALIGN(4);

. = . + 0x1000;

_stack_top = .;		/*stack top after 4 KB*/

}>SRAM

}

# Map file

Memory Configuration

Name             Origin             Length             Attributes

FLASH            0x08000000         0x00010000         xr

SRAM             0x20000000         0x00005000         xrw

*default*        0x00000000         0xffffffff

Linker script and memory map

.text           0x08000000      0x1f8

*(.vectors*)

.vectors       0x08000000       0x1c startup.o

0x08000000                vectors

*(.text*)

.text          0x0800001c       0x64 Alarm.o

0x0800001c                Alarm_Init

0x08000028                Set_Alarm_actuator

0x08000064                StartAlarm

0x08000072                StopAlarm

.text          0x08000080       0x38 Pressure_Sensor.o

0x08000080                PressureSensor_Init

0x08000098                getPressureVal

.text          0x080000b8       0x74 driver.o

0x080000b8                Delay

0x080000da                GPIO_INITIALIZATION

.text          0x0800012c       0x34 main.o

0x0800012c                main

.text          0x08000160       0x98 startup.o

0x08000160                Reset_Handler

0x080001ec                MM_fault_Handler

0x080001ec                Usage_fault_Handler

0x080001ec                Bus_fault_Handler

0x080001ec                Default_Handler

0x080001ec                H_fault_Handler

0x080001ec                NMI_Handler

*(.rodata)

0x080001f8                . = ALIGN (0x4)

0x080001f8                _E_text = .

.glue_7         0x080001f8        0x0

.glue_7        0x080001f8        0x0 linker stubs

.glue_7t        0x080001f8        0x0

.glue_7t       0x080001f8        0x0 linker stubs

.vfp11_veneer   0x080001f8        0x0

.vfp11_veneer  0x080001f8        0x0 linker stubs

.v4_bx          0x080001f8        0x0

.v4_bx         0x080001f8        0x0 linker stubs

.iplt           0x080001f8        0x0

.iplt          0x080001f8        0x0 Alarm.o

.rel.dyn        0x080001f8        0x0

.rel.iplt      0x080001f8        0x0 Alarm.o

.data           0x20000000        0x0 load address 0x080001f8

0x20000000                _S_data = .

*(.data)

.data          0x20000000        0x0 Alarm.o

.data          0x20000000        0x0 Pressure_Sensor.o

.data          0x20000000        0x0 driver.o

.data          0x20000000        0x0 main.o

.data          0x20000000        0x0 startup.o

0x20000000                . = ALIGN (0x4)

0x20000000                _E_data = .

.igot.plt       0x20000000        0x0 load address 0x080001f8

.igot.plt      0x20000000        0x0 Alarm.o

.bss            0x20000000     0x1004 load address 0x080001f8

0x20000000                _S_bss = .

*(.bss*)

.bss           0x20000000        0x0 Alarm.o

.bss           0x20000000        0x4 Pressure_Sensor.o

0x20000000                g_pressureVal

.bss           0x20000004        0x0 driver.o

.bss           0x20000004        0x0 main.o

.bss           0x20000004        0x0 startup.o

0x20000004                _E_bss = .

0x20000004                . = ALIGN (0x4)

0x20001004                . = (. + 0x1000)

*fill*         0x20000004     0x1000

0x20001004                _stack_top = .

LOAD Alarm.o

LOAD Pressure_Sensor.o

LOAD driver.o

LOAD main.o

LOAD startup.o

OUTPUT(Pressure_Controller.elf elf32-littlearm)

LOAD linker stubs

.debug_info     0x00000000      0x4be

.debug_info    0x00000000       0xf9 Alarm.o

.debug_info    0x000000f9       0xcc Pressure_Sensor.o

.debug_info    0x000001c5       0xc8 driver.o

.debug_info    0x0000028d       0x9c main.o

.debug_info    0x00000329      0x195 startup.o

.debug_abbrev   0x00000000      0x28d

.debug_abbrev  0x00000000       0x80 Alarm.o

.debug_abbrev  0x00000080       0x66 Pressure_Sensor.o

.debug_abbrev  0x000000e6       0x7c driver.o

.debug_abbrev  0x00000162       0x59 main.o

.debug_abbrev  0x000001bb       0xd2 startup.o

.debug_loc      0x00000000      0x2b0

.debug_loc     0x00000000       0xe0 Alarm.o

.debug_loc     0x000000e0       0x88 Pressure_Sensor.o

.debug_loc     0x00000168       0xa0 driver.o

.debug_loc     0x00000208       0x2c main.o

.debug_loc     0x00000234       0x7c startup.o

.debug_aranges  0x00000000       0xa0

.debug_aranges

0x00000000       0x20 Alarm.o

.debug_aranges

0x00000020       0x20 Pressure_Sensor.o

.debug_aranges

0x00000040       0x20 driver.o

.debug_aranges

0x00000060       0x20 main.o

.debug_aranges

0x00000080       0x20 startup.o

.debug_line     0x00000000      0x4b4

.debug_line    0x00000000      0x10b Alarm.o

.debug_line    0x0000010b      0x101 Pressure_Sensor.o

.debug_line    0x0000020c      0x106 driver.o

.debug_line    0x00000312       0x4e main.o

.debug_line    0x00000360      0x154 startup.o

.debug_str      0x00000000      0x24b

.debug_str     0x00000000      0x14a Alarm.o

0x18c (size before relaxing)

.debug_str     0x0000014a       0x43 Pressure_Sensor.o

0x194 (size before relaxing)

.debug_str     0x0000018d       0x2a driver.o

0x17b (size before relaxing)

.debug_str     0x000001b7        0xc main.o

0x157 (size before relaxing)

.debug_str     0x000001c3       0x88 startup.o

0x1dc (size before relaxing)

.comment        0x00000000       0x49

.comment       0x00000000       0x49 Alarm.o

0x4a (size before relaxing)

.comment       0x00000049       0x4a Pressure_Sensor.o

.comment       0x00000049       0x4a driver.o

.comment       0x00000049       0x4a main.o

.comment       0x00000049       0x4a startup.o

.ARM.attributes

0x00000000       0x2d

.ARM.attributes

0x00000000       0x2d Alarm.o

.ARM.attributes

0x0000002d       0x2d Pressure_Sensor.o

.ARM.attributes

0x0000005a       0x2d driver.o

.ARM.attributes

0x00000087       0x2d main.o

.ARM.attributes

0x000000b4       0x2d startup.o

.debug_frame    0x00000000      0x1b0

.debug_frame   0x00000000       0x8c Alarm.o

.debug_frame   0x0000008c       0x50 Pressure_Sensor.o

.debug_frame   0x000000dc       0x58 driver.o

.debug_frame   0x00000134       0x2c main.o

.debug_frame   0x00000160       0x50 startup.o

# Simulation